Stochastic control

Introduction

Why is it interesting?

Examples.

- Admitting connections in an ATM network.

- New connections arrive (Poisson Process)

- If admitted, they alternate between “Active” and “Idle.”

- When active, generate a constant stream of  “packets.”

- Channel has limited capacity, no buffering: too many streams imply loss of packets.

- QoS = Quality of Service: want small losses, and small probability of blocking.

- Goals are conflicting: best policy for one is worst for the other.

- Formulate optimization problem: steady state, short term, long term.

- Multi-tasking

- Discrete time model

- K different types of jobs, each with its own buffer, single processor

- Geometric service times (that is, no memory)

- “Penalty:” for waiting time

- Minimize time-and-ensemble average.

- Observations:

- if all penalties are equal, priority to faster jobs.

- if all rates are equal, priority of jobs with high penalty.

- Challenge: is it optimal in general to use priorities? How to compute?

- Routing

- Requests of connections arrive (Poisson)

- There are only two lines: fast and slow (exponential)

- Penalty: total waiting time including service time (!)

- Should anyone be routed through the slow connection?

- Observations:

- If the line is very long, yes: will complete slow service before all are

served by fast connection.

- If there is only one job in queue, send to fast connection.

- “Obvious:” threshold.

We will formulate a general class of criteria, and derive methods for finding optimal policies.

The goals:

- formulate models

- formulate criteria

- find optimal policies under criteria: structure, explicit calculation.

- Develop mathematical tools and skills

- Develop software tools and skills

- Develop intuition concerning network behavior and optimization

Administration:

- Homework (theoretical)

- Computer assignments

- Project (theoretical/computer).

Dynamic programming – intuition:

Challenge 1.

Start

End

At each step move to the right.

- How many possible paths?

- How many paths in N steps, if each step involves k options?

Suppose each link carries a cost. Objective: find a path with minimum cost.

- How many additions and how many binary comparisons, to find the best?

- How much memory?

- Prove the principle of optimality: if  p  is an optimal path (length N), then the last k steps of the path are an optimal path from its starting point to the end.

- How would things change if, when you choose a path segment, with probability 0.1 you end up on the other segment? Is optimality possible?

Example of solution methodology: dynamic programming.

To solve the problem of the challenge, use backward induction and the principle of optimality:

- find the optimal one-step path for each terminal state. Denote its cost by V1(x)

- find the optimal one-step path from two stages before end to one stage, with “terminal cost” V1(x). Denote its cost by V2(x)

- find the optimal one-step path from three stages before end to two stages, with “terminal cost” V2(x). Denote its cost by V3(x). Repeat.

- How many additions and how many binary comparisons, to find the best?

- How much memory?

Goals of the course:

- Derive (stochastic) models, mostly applications from communications and digital networks

- Criteria for performance: standard and otherwise

- General tools (algorithmic) for solving such optimization problems

- Special models and special tools

- Computer tools: existing, development